BrailleNote Printer Settings
To set BrailleNote for embosser.

1) Go to the Main Menu

2) Press W for word processor

3) Press P for printing
4) Press S for setup.

Space through the options and check they are as follows. If any setting needs to be changed either type in the number required (number sign is not required) or initial letter of setting required. Then press enter.

Paper length – 66
Paper Width – 85
Form Feed – No

Pause – No

Page Offset - 0
Printer Port – USB
Printer Type – Standard
Extended Character Set – MS-DOS for English only printing or International MS-DOS if French is required.

At the ‘end of list’  press ‘space & e’ to exit. You will be prompted if you want to save changes ‘y’ for yes, ‘n’ for no.

Then ‘space & e’ back up the menu levels.

